Now we'll consider another new operation called multi head attention MHA, it contains 5 matrix multiplication steps, I'll explain it to you.

First step is at which we get Q, K and V matrices, first the image is loaded from the output memory by dimensions (56x56,96), this image will be chopped into smaller non overlapping windows of 7x7 window, so the hardware takes a single 7x7 chunk of that 56x56 image and it flattens that 7x7 grid into a sequence of 49 patches, since each patch still has 96 channels, the actual input matrix being fed into the hardware for the next step is 49x96, so we have totally 64 chunk of 49x96 as the total size image is 56x56,96, okay.

Now we'll go to first step to calculate Q, K and V matrices, knowing that we calculate only one at a time in the given MMU so we calculate Q first then K then V okay.

Let say we'll calculate Q firstly, so the input first chunk of 49x96 will go into the MMU and the weight matrix of dimension 96x96 will go too into the MMU to produce the 49x96 output matrix of this first chunk. The matrix multiplication will be as following:

We have 12 PEs in the MMU, each PE takes 4 different weights and 28 different data window.

If we do matrix Multiplication between 49x96 and 96x96, the output looks like that: 
Oi,j=k=095Ii,k⋅Wk,j

O0×0=I0×0W0×0+I0×1W1×0+I0×2W2×0+I0×3W3×0+…+I0×95W95×0

O1×0=I1×0W0×0+I1×1W1×0+I1×2W2×0+I1×3W3×0+…+I1×95W95×0

So the first PEs will take W0×0 , W1×0 , W2×0 and W3×0 as weights and take I0×0 I0×1 I0×2 I0×3 of O0×0 and take I1×0 I1×1 I1×2 I1×3 of O1×0 and  so on till take I6×0 I6×1 I6×2 I6×3 of O6×0, so 28 input data I and 4 weights.

the second PEs will take W4×0 , W5×0 , W6×0 and W7×0 as weights and take I0×4 I0×5 I0×6 I0×7 of O0×0 and take I1×4 I1×5 I1×6 I1×7 of O1×0 and  so on till take I6×4 I6×5 I6×6 I6×7 of O6×0, so 28 input data I and 4 weights.

And so on for all PEs

the last (12) PEs will take W44×0 , W45×0 , W46×0 and W47×0 as weights and take I0×44 I0×45 I0×46 I0×47 of O0×0 and take I1×44 I1×45 I1×46 I1×47 of O1×0 and  so on till take I6×44 I6×45 I6×46 I6×47 of O6×0, so 28 input data I and 4 weights.

Knowing that all the 12 PEs are working in parallel.

We don’t care about how these operations done in the MMU but we’re care about how the output buffer and the output memory handled these data.

So, A 7x1 output vector (O0×0, O1×0 O2×0 O3×0O4×0O5×0O6×0) is produced each two cycles, so after first 2 cycles we’ll get first 7 elements of the output Q matrix, so we need 14 cycles to finish the first column of the output matrix from O0×0 to O48×0, as the output matrix contains 49x96, so we’ll need to wait 14x32=1344 cycles to finish the 49x96 output matrix from first chunk of the data, the output first chunk of 49x96 of Q will be stored in the output memory.

Then this Q matrix of 49x96, will be splitted into 3(number of heads) matrices of size 49x32 in the output memory.

Now we’ll go to second matrix multiplication to get K matrix, it’s the same operation and calculations I explained above but before it, we’ll need to load the new weights of K matrix, and so on for getting V matrix, knowing that they are chunks of 49x96 and all are stored in the output memory but in different locations, so 1344 cycles to finish V and 1344 cycles to finish K

So now we have 4032 cycles to finish generating Q,K,V

Then K and V matrices of 49x96, will be splitted into 3(number of heads) matrices of size 49x32 in the output memory, so now we have 3 matrices of 49x32 of V and same for Q and same for K, so totally 9 matrices of 49x32 in the output memory.

This splitting operation should be done in the output memory for example splitting the 49x96 matrix into three small matrix of 49x32 as three columns.

After we’ll get Q,K and V matrices(3 matrices of 49x32 of V and same for Q and same for K) and all are stored in the output memory, we will go to Attention score step. Attention score step is just doing matrix multiplication of QxKT, knowing that we will have to transpose the K matrix (transpose each one of the three K of 49x32 matrices) that are calculated and stored in the output memory, we need to transpose them (three matrices), I don’t know how but somehow it should be done in the memory by I think re-ordering the output K data from the output memory into the Data selection unit in the input of the MMU, however I trust on you to make it, after you transpose them, load one of them (don’t forget we have 3 of 49x32 size) in the Weight buffer (the weight input buffer) and load also the Q matrix of 49x32(load one of them (don’t forget we have 3 of 49x32 size) that are stored in the output memory too, load the Q of 49x32 in the unified input buffer, so the output expected to be 49x49, the matrix multiplication here will be 49x32 into 32x49 so, A 7x1 output vector (O0×0, O1×0 O2×0 O3×0O4×0O5×0O6×0) is produced each one cycle, so we need 7 cycles to finish only one column so we need 7x49=343 cycles to finish the output QxKT  (first output from the three)chunk matrix, then it will be stored in the output memory, then we will do the same operation for the second and third K and Q of 49x32, then store the results in the output memory, so we have 343 x 3 = 1029 cycles to finish their operations, then all must be stored in the output memory through the small output buffer

After we’ll get QxKT chunk matrices(3 matrices of 49x49 in 1029 cycles) and are stored in the output memory, we’ll go to S x V step. This is a matrix multiplication of S(3 matrices of 49x49 which are resulted from QxKT and are stored in the output memory) and V (3 matrices of 49x32 which are calculated in the Q,K, V matrix multiplication step and stored here) so we expect the output of size 49x32(3 matrices of 49x32).

Oi,j=k=049Si,k⋅Vk,j

O0×0=S0×0V0×0+S0×1V1×0+S0×2V2×0+S0×3V3×0+…+S0×47V47×0+S0×48V48×0

O1×0=S1×0V0×0+S1×1V1×0+S1×2V2×0+S1×3V3×0+…+S1×47V47×0+S1×48V48×0

S x V multiplication is like and same before we did and a full 7x1 output vector is ready every 2 cycles, but this raises a serious utilization problem as in the first cycle the 12 PE blocks are fully utilized producing 48 partial sums, the second cycle only a single column in a single PE block is utilized, so we have 224 cycles to finish one of 49x32 thus we need 224 x 3 = 672 cycles to finish S x V.

Knowing that our Processing order is Feeding rows  Exporting row .

After finished did the SxV multiplication, we’ll store the outcome in the output memory, so now we finished the Multi-Head Attention of the first chunk which was 49x96, then we’ll re-do all previous calculations again for second chunks of 49x96 of the data image and so on, knowing that we have 64 chunks of 49x96 of data image

re-modify the output memory such that its name is intermediate buffer not output memory as it holds all the temporary intermediate data after the MMU. intermediate buffer contains 1 memory layer for patch embedding, 1 memory layer for patch merging, 5 memory layers for MSA (layer for Q matrix: Q = X x WQ, layer for K matrix: K = X x Wk, layer for V matrix: V = X x WV, layer for Attention score: QxKT, layer for S x V) and 1 memory layer for patch merging operation, 1 memory layer for MLP. we'll focus now to update the memory layer of patch embedding. in patch embedding we do convolution (224,224,3) with 96 kernels of (4,4,3) to produce (56,56,96). details of this operation: "1) Convolution Layer:

The convolution operation is done only once in the patch embedding stage at the first layer, which uses a 4x4 convolution with stride 4 to reduce the input size to 1/4 of its input size. The Kernel size is 4x4x3.

The convolution can be perceived as an Element-wise Multiplication, where each weight in the kernel is multiplied by the corresponding pixel it currently overlaps. Then all these products are summed together to produce a single value.

We have 12 PE blocks, each 4 blocks are responsible for an input channel, the 7 rows of each block are active at the same time producing 7 outputs each cycle.

To produce one output channel, the Kernel needs to slide through the whole 224x224x3 image, thus in order to finish four complete rows, it requires , to finish the whole image, it requires 448 cycles.

The computing order of the convolution will process the first output channel then the second one until the final 96th channel, this will require 96x488 = 46,848 cycles.

As discussed before, 7 complete outputs are generated every cycle, the first element being the summation of “Buffer 0” of all the 12 PE blocks, the second element being the summation of “Buffer 1” and so on, the second cycle processes the next 7 columns [7:13], completing 4 full rows every 8 cycles.". thus the intermediate buffer should take the 7 output elements from the output buffer and store it row by row, for example, after the first cycle we have v1 v2 v3 v4 v5 v6 v7, the second cycle we have v8 v9 v10 v11 v12 v13 v14 and so on untill we reach v56, so now we complete full complete row in 8 cycles then we shift into the second row and repeat this and so on, till the we complete first 56x56, then we re-do the same flow for the second 56x56 and so on untill we complete the convolution so we have 56x56,96. i want the storing is continous in memory as the first element is v1 then v2 and so on till v3136 this for the first channel, the second channel its address will start from v3137 till v6272 and so on till we reach to the 96th channel. in patch merging process, it has 2 operations, one should be done by the MMU and the other by the memory. the first operation is "The Spatial Merge (Concatenation)", the patch merging memory layer takes the data which is (56x56,96) from the MLP memory layer then firstly it will do Spatial Merge operation by The goal here is to reduce the spatial resolution by half while preserving all the feature information by doing Grid Grouping: The network looks at the $56 \times 56$ spatial grid and divides it into $2 \times 2$ blocks. Since $56 / 2 = 28$, this results in a new spatial grid of $28 \times 28$, Channel Stacking: Inside every single $2 \times 2$ block, there are 4 individual patches. Currently, each patch is represented by a $96$-dimensional vector (since $C = 96$), The Math: The operation concatenates the 4 vectors in each block together.

$4 \text{ patches} \times 96 \text{ channels} = 384 \text{ channels}$. thus At this exact moment, our tensor has a shape of $28 \times 28 \times 384$. We have successfully traded spatial resolution for channel depth. the second operation is we do fully connected layer to convert (28x28,384) into (28x28,192), so we should get out stored (28x28,384) to some off-chip memory (don't care about this just take care this data will get out laterlly).

make sizing table, sizing the input data, input weight and input bias and output final value for each stage of this swin architecture. the flow is: Patch embedding (convolution) (input data is (224, 224, 3), input weight 96 kernels of size(4, 4,3) and 96 bias to produce output of (56x56,96)) then W-MSA then MLP (2 fully connected layer: the first layer takes input data of size (56x56,96), input weight(96,384), input bias is 384 value then produce output (56x56,384), the second layer takes input data of size (56x56,384), input weight(384,96), input bias is 96 value then produce output of size (56x56,96)) (W-MSA + MLP are first swin transformer block of first stage) then the second swin transformer block of the first stage (SW-MSA + MLP) then patch merging of the second stage (spatial merging(convert (56x56,96) into (28x28,384)) + FC layer which takes input data size (28x28,384) and weight data size (384,192) and 192 bias value to produce (28x28,192)). Then again first swin transformer block of second stage then second swin transformer block of second stage then patch merging of the third stage. After that first swin transformer block of third stage then second swin transformer block of third stage then third swin transformer block of third stage and so on till 6th swin transformer block of third stage then patch merging of the fourth stage. then first swin transformer block of fourth stage then second swin transformer block of fourth stage then finally a single fully connected layer.